Self-Balancing Two-Wheel Robot Using PID Control

SUBMITTED BY:

Ali Ahsan	2023-EE-173

Abdul Muiz 2023-EE-162

Abdul Basit 2023-EE-167

SUBMITTED TO:

Department of Electrical Engineering

University of Engineering and Technology (UET)

Habib Wajid 
Self-Balancing Robot with Light Following (MIMO Control)

# 1. Objective

The objective of this experiment was to design and implement a self-balancing robot capable of following a light source while maintaining dynamic stability. The system combines:

- A fast inner-loop PID controller for balancing

- A slow outer-loop PID controller for light-following

- Multi-rate MIMO (Multi-Input Multi-Output) control architecture

- Real-time telemetry and WiFi dashboard monitoring

The robot successfully demonstrates simultaneous stabilization and navigation behavior using hierarchical control principles.

# 2. Introduction

A self-balancing robot behaves like an inverted pendulum system, which is naturally unstable. To maintain balance, continuous feedback from an IMU sensor is required. In this experiment, additional light sensors (LDRs) were integrated so the robot could detect and move toward a light source.

The implemented system contains two interacting control loops:

| Control Loop | Purpose | Frequency |
| --- | --- | --- |
| Inner Loop | Balance stabilization | 100 Hz |
| Outer Loop | Light-following direction control | 20 Hz |

This follows the frequency-decoupling principle described in the lab manual.

# 3. System Overview

The robot consists of the following major subsystems:

- ESP32 microcontroller

- MPU6050 IMU sensor

- TB6612FNG motor driver

- Two DC geared motors

- Three LDR light sensors

- WiFi-based telemetry dashboard

- Dual PID control architecture

The system uses:

- IMU data for tilt-angle estimation

- LDR sensor readings for light-direction estimation

- Motor differential control for turning

# 4. Hardware Components

| Component | Purpose |
| --- | --- |
| ESP32 | Main controller |
| MPU6050 | Tilt angle measurement |
| TB6612FNG | Dual motor driver |
| DC Motors | Robot movement |
| LDR Sensors | Light detection |
| Li-ion Battery | Power supply |
| WiFi Dashboard | Real-time monitoring |

The lab manual required at least two light sensors, while this implementation used a three-sensor arrangement (Left, Center, Right) for improved directional accuracy.

# 5. System Architecture

## Block Diagram

The architecture follows the hierarchical control model specified in the lab manual.

# 6. Software Design

The software was implemented in Arduino C++ using ESP32 libraries.

Main libraries used:

#include <Wire.h>

#include <math.h>

#include <WiFi.h>

#include <WebServer.h>

The software contains:

- WiFi server

- Sensor acquisition

- Complementary filter

- Balance PID controller

- Light-following PID controller

- Real-time dashboard

- Telemetry system

# 7. MPU6050 Angle Estimation

The MPU6050 provides:

- Accelerometer data

- Gyroscope data

A complementary filter was used to estimate robot angle.

The accelerometer angle is computed as:

The complementary filter equation:

This filter combines:

- Fast gyroscope response

- Long-term accelerometer stability

Result:

- Smooth angle estimation

- Reduced sensor noise

- Stable balancing performance

# 8. Inner Loop – Balance PID Controller

The inner loop stabilizes the inverted pendulum system.

The implemented PID equation is:

| Variable | Description |
| --- | --- |
| (e) | Balance error |
| (K_p) | Proportional gain |
| (K_i) | Integral gain |
| (K_d) | Derivative gain |

Implemented gains:

float Kp = 12;

float Ki = 0.2;

float Kd = 1;

The loop executes every 10 ms (100 Hz).

Functions performed:

- Read IMU

- Estimate angle

- Compute PID

- Generate balance command

- Drive motors

The balance controller remained the primary fast subsystem as required in the lab manual.

# 9. Outer Loop – Light Following PID

The outer loop controls robot direction based on light intensity.

The light error is computed as:

e_{light}=L_{left}-L_{right}

Interpretation:

| Condition | Meaning |
| --- | --- |
| (e_{light} > 0) | Light on left |
| (e_{light} < 0) | Light on right |
| (e_{light} = 0) | Robot aligned |

The directional PID controller is:

Implemented gains:

float KpL = 0.050;

float KiL = 0.0;

float KdL = 0.006;

The outer loop runs every 50 ms (20 Hz).

This slower operation prevents disturbance to the balance loop.

# 10. MIMO Control Integration

The two control loops are combined using motor mixing equations:

u_{left}=u_{bal}+u_{dir}

u_{right}=u_{bal}-u_{dir}

Effects:

| Motion Type | Cause |
| --- | --- |
| Forward/Backward | Balance controller |
| Turning | Light controller |

This structure allows simultaneous balancing and steering.

# 11. Multi-Rate Control Implementation

Two timing loops were implemented:

## Fast Loop (100 Hz)

Tasks:

- Read MPU6050

- Estimate angle

- Compute balance PID

- Drive motors

Code timing:

if (now - lastFast >= 10)

## Slow Loop (20 Hz)

Tasks:

- Read LDR sensors

- Compute light error

- Compute directional PID

- Generate turn command

Code timing:

if (now - lastSlow >= 50)

This implementation matches the multi-rate architecture described in the lab manual.

# 12. Forward Motion Strategy

An additional feature was implemented using the center LDR sensor.

When the center sensor detects the brightest light:

forwardBoost = 1.2;

This slightly shifts the balance angle forward, causing the robot to move toward the light source while remaining balanced.

Advantages:

- Smooth forward motion

- Improved tracking behavior

- Reduced unnecessary turning

# 13. WiFi Dashboard

A real-time dashboard was developed using:

- HTML

- CSS

- JavaScript

- ESP32 WebServer

Dashboard features:

- Live angle display

- PID parameter tuning

- Motor output monitoring

- LDR sensor visualization

- Telemetry streaming

- Real-time control updates

# 14. Experimental Results

The robot successfully demonstrated:

- Stable balancing

- Directional turning toward light

- Real-time telemetry

- WiFi control

- Simultaneous balance and navigation

- Smooth turning behavior

- Forward movement toward aligned light

| Light Position | Robot Response |
| --- | --- |
| Left | Turn left |
| Right | Turn right |
| Center | Move forward |
| No light | Maintain balance |

# 15. Tuning Procedure

## Inner Loop Tuning

The balance PID was tuned first:

- Increase (K_p)

- Add (K_d) for damping

- Add small (K_i) to reduce steady-state error

Final tuned values:

Kp = 18.1

Ki = 0.16

Kd = 2.7

## Outer Loop Tuning

The light PID was tuned second:

- Start with small (K_p^L)

- Increase until turning begins

- Add (K_d^L) to reduce oscillation

- Keep gains small

Final values:

KpL = 0.050

KiL = 0.0

KdL = 0.006

# 16. Challenges Faced

Several challenges were encountered during implementation:

## Sensor Noise

- MPU6050 gyroscope drift

- LDR fluctuations due to ambient light

## Stability Issues

- Excessive turning disturbed balance

- Large forward boost caused instability

## PID Interaction

- Strong outer-loop gains affected inner-loop stability

- Careful frequency separation was necessary

## Mechanical Vibrations

- Motor vibrations introduced noise into IMU readings

# 17. Discussion

The experiment successfully demonstrated a practical MIMO control system.

Key observations:

- Frequency decoupling simplified controller design

- Fast balance loop maintained stability

- Slow directional loop enabled navigation

- Small outer-loop gains prevented instability

- Hierarchical control improved robustness

The project clearly showed the transition from:

- Single-loop control
to

- Multi-objective robotic control

This aligns with the learning outcomes described in the lab manual.

# 18. Conclusion

A self-balancing light-following robot was successfully designed and implemented using ESP32 and MPU6050.

The system achieved:

- Stable balancing

- Light tracking

- Smooth directional control

- Multi-rate PID control

- Real-time telemetry monitoring

The experiment demonstrated:

- Practical PID implementation

- MIMO control architecture

- Frequency-decoupled control

- Real-time embedded systems integration

The robot successfully fulfilled the objectives and requirements of Lab 13.